.

.

.

.

.

.

.

.

Ans 36.

| Feature | MQTT | HTTP |
| --- | --- | --- |
| Architecture Pattern | Publish/Subscribe (Broker-based) | Client-Server |
| Data Transfer Model | Devices publish messages to a broker, subscribers receive them | Client sends request, server sends response |
| Power Consumption | Low | High |
| Latency | Low (Fast communication) | Higher than MQTT |
| IoT Suitability | Best for IoT devices and real-time communication | Better for websites, APIs, and web applications |
| Security Considerations | Supports SSL/TLS, username/password authentication | Supports HTTPS (SSL/TLS), authentication, and encryption |

Protocol I will choose will be MQTT

### Reason:

- Low power consumption, which helps battery-powered sensors.

- Low bandwidth usage, making communication efficient.

- Fast data transfer with low latency.

- Can handle a large number of IoT devices efficiently.

- Reliable communication using Quality of Service (QoS).

Ans 37. Wi-Fi security is very important in IoT because devices such as ESP32 send and receive data over wireless networks. If the network is not secure, attackers can steal data or gain unauthorized access. Two common Wi-Fi attack vectors are WPA2 vulnerabilities and Evil Twin attacks. Some older WPA2 implementations are vulnerable to security weaknesses if devices are not updated. In an Evil Twin attack, an attacker creates a fake Wi-Fi network with the same name (SSID) as the real one. IoT devices may connect to the fake network, allowing the attacker to intercept or modify data.

To improve ESP32 Wi-Fi security, developers should use WPA3 whenever it is available because it provides stronger authentication and better protection than WPA2. Another good practice is certificate pinning, where the ESP32 verifies the server’s trusted certificate before sending data. This helps prevent man-in-the-middle attacks. Developers should also use HTTPS/TLS encryption, keep firmware updated, and use strong, unique Wi-Fi passwords. Disabling unused services and enabling secure boot and flash encryption (when supported) further improves security.

Hardcoding Wi-Fi passwords or API keys directly in the source code is dangerous. If the code is uploaded to GitHub or shared with others, the credentials can be exposed. Attackers may then access the Wi-Fi network or cloud services. A better approach is to store sensitive information in a separate configuration file (such as config.h), exclude it from GitHub using .gitignore, or use secure storage mechanisms.

By following these security practices, ESP32-based IoT devices become more secure, reliable, and resistant to common wireless attacks.

Ans 38. LoRa (Long Range) is a wireless communication technology designed for IoT applications that require long-distance communication with very low power consumption. It is based on LPWAN (Low Power Wide Area Network), which is a type of network that allows devices to transmit small amounts of data over several kilometers while using very little battery power. LPWAN is ideal for applications such as agriculture, smart cities, environmental monitoring, and asset tracking.

LoRa achieves long-range communication using a technique called the Spreading Factor (SF). A higher spreading factor spreads the signal over a longer period, making it easier for the receiver to detect weak signals. This increases communication range and reliability, although the data transmission speed becomes slower. Lower spreading factors provide faster data rates but shorter communication distances.

### Comparison of LoRa, Zigbee, and Z-Wave

| Feature | LoRa | Zigbee | Z-Wave |
| --- | --- | --- | --- |
| Range | 2–15 km | 10–100 m | 30–100 m |
| Power Consumption | Very Low | Low | Low |
| Data Rate | Low | High | Medium |
| Best Use | Agriculture, Smart Cities | Home Automation | Smart Home Devices |

A real-world example in India is the use of LoRa-based smart agriculture systems in states such as Karnataka and Andhra Pradesh, where soil moisture and weather sensors send data over long distances to help farmers monitor crops and reduce water usage.

Ans 39. An IoT Gateway is a device that acts as a bridge between IoT sensor nodes and the cloud. It collects data from sensors, processes or filters it if required, converts different communication protocols, and securely sends the data to the cloud. It also receives commands from the cloud and forwards them to IoT devices.

### Simple Diagram

+-------------+     +-------------+     +-------------+     +----------------+
| IoT Sensors | --> | IoT Gateway | --> | Cloud Server| --> | User App/Web   |
+-------------+     +-------------+     +-------------+     +----------------+

### Protocols Used at Each Layer

| Layer | Common Protocols |
| --- | --- |
| IoT Sensor Nodes → Gateway | Zigbee, LoRa, BLE, Wi-Fi |
| Gateway → Cloud | MQTT, HTTP, HTTPS |
| Cloud → User Application | HTTPS, REST API, WebSocket |

### Ans 40. QoS Levels

| QoS Level | Meaning | Description |
| --- | --- | --- |
| QoS 0 | At most once | Message is sent only once. No confirmation is received. |
| QoS 1 | At least once | Message is guaranteed to arrive, but it may be delivered more than once. |
| QoS 2 | Exactly once | Message is delivered only once with the highest reliability. |

### Which QoS is Best for an IoT Water Level Monitoring System?

Answer: QoS 1 (At least once)

Reason:

- Ensures that important water level data reaches the server.

- Slight duplicate messages are acceptable.

- Lower overhead than QoS 2.

- More reliable than QoS 0.

- Commonly used in IoT monitoring applications.

Ans 42 CODE-

Start

Initialize ESP32

Initialize Soil Moisture Sensor

Initialize LDR Sensor

Initialize Relay (Pump)

Initialize Push Button

Initialize LCD

Connect to Wi-Fi

Loop

Read Soil Moisture Value

Read LDR Value

If Manual Button is Pressed

Turn Pump ON

Wait 5 Minutes

Turn Pump OFF

Else

If Soil is Dry

If LDR Detects Rain (Low Light)

Pump OFF

Else

Pump ON

End If

Else

Pump OFF

End If

End If

Display Moisture Percentage

Display Pump Status

Display Last Watered Time

Upload Data to Google Sheets

Wait 30 Minutes

Repeat

FLOWCHART-

START

│

▼

Initialize ESP32

│

▼

Read Soil Moisture Sensor

│

▼

Read LDR Sensor

│

▼

Is Manual Button Pressed?

│              │

Yes             No

│              │

▼              ▼

Turn Pump ON     Check Soil Moisture

for 5 Minutes          │

▼

Is Soil Dry?

│         │

No         Yes

│          │

▼          ▼

Pump OFF   Is It Dark

(Possible Rain)?

│          │

Yes         No

│           │

▼           ▼

Pump OFF     Pump ON

│           │

└──────┬────┘

▼

Display Moisture, Mode &

Last Watered Time on LCD

│

▼

Send Data to Google Sheets

│

▼

Repeat Loop

Ans 41.                     START

│

▼

Initialize ESP32

│

▼

Initialize MQ-2, DHT11,

RGB LED, Buzzer & Bluetooth

│

▼

Read MQ-2 Gas Sensor Value

│

▼

Read Temperature & Humidity

from DHT11 Sensor

│

▼

Compare Gas Value with

Predefined Thresholds

│

┌───────────┼───────────┐

▼           ▼           ▼

SAFE      MODERATE     DANGER

│           │           │

Green LED ON   Yellow LED ON  Red LED ON

Short Beep     Medium Beep    Long Beep

│           │           │

└───────────┼───────────┘

▼

Display Data on Serial Monitor

(CSV Format with Timestamp)

│

▼

Send Alert to Mobile via

Bluetooth (if Warning/Danger)

│

▼

Wait 2 Seconds

│

▼

Repeat Loop

Code-

StartInitialize ESP32

Initialize MQ-2 Sensor

Initialize DHT11 Sensor

Initialize RGB LED

Initialize Buzzer

Initialize Bluetooth

Loop Forever

Read MQ-2 Gas Value

Read Temperature

Read Humidity

If Gas Value < Safe Threshold

Turn ON Green LED

Turn OFF Yellow & Red LED

Buzzer OFF

Severity = SAFE

Else If Gas Value < Danger Threshold

Turn ON Yellow LED

Turn OFF Green & Red LED

Play Medium Beep

Severity = MODERATE

Else

Turn ON Red LED

Turn OFF Green & Yellow LED

Play Long Beep

Severity = DANGER

End If

Print Timestamp

Print Gas Value

Print Temperature

Print Humidity

Print Severity

If Severity is MODERATE or DANGER

Send Bluetooth Alert

End I

Wait 2 Seconds

Repeat

Sample serial monitor output-

Timestamp,MQ2 Value,DHT11 Temp,DHT11 Humidity,Severity

00:01,220 ppm,23.4°C,49%,SAFE

00:02,280 ppm,23.5°C,50%,SAFE

00:03,420 ppm,23.6°C,49%,MODERATE

00:04,710 ppm,23.8°C,48%,DANGER

Sample mobile alert-

SAFE Gas: 210 ppm

Temperature: 22.8°C

Humidity: 51%

Status: SAFE

WARNING Moderate Smoke Detected

Gas: 430 ppm

Check Fireplace

DANGER

Smoke Level Critical

Gas: 765 ppm

Open Windows

Leave Room if Necessary

Ans 43.

START

│

▼

Initialize ESP32

│

▼

Initialize Keypad, LCD/OLED,

Bluetooth and Servo Motor

│

▼

User Enters 4-digit PIN

│

▼

Is PIN Correct?

│          │

No         Yes

│          │

▼          ▼

Display Wrong   Generate

PIN         6-digit OTP

│          │

▼          ▼

Log Failure   Send OTP via

Bluetooth

│

▼

Start 30-sec Timer

│

▼

User Enters OTP

│

┌─────────┴─────────┐

▼                   ▼

OTP Correct & Time       Wrong OTP/

≤30 Seconds          Timer Expired

│                   │

▼                   ▼

Rotate Servo 90°        Access Denied

Display Access          Log Failure

Granted

│

▼

Log Success

│

▼

Repeat

Start

Initialize ESP32

Initialize Keypad

Initialize LCD

Initialize Bluetooth

Initialize Servo

Loop

Read 4-digit PIN

If PIN is Correct

Generate Random 6-digit OTP

Send OTP via Bluetooth

Start 30-second Timer

Read OTP from User

If OTP is Correct AND Time < 30 Seconds

Rotate Servo to 90 Degrees

Display "Access Granted"

Log Success to Serial Monitor

Else

Display "Access Denied"

Log Failure to Serial Monitor

End If

Else

Display "Wrong PIN"

Log Failure to Serial Monitor

End If

Repeat

Sample serial monitor output

Time,PIN,OTP,Status

10:00:15,1234,654321,ACCESS GRANTED

10:05:40,1234,125678,WRONG OTP

10:10:12,9999,------,WRONG PIN

10:15:50,1234,Expired,OTP TIMEOUT

Sample mobile alert

SMART DOOR LOCK

Your OTP is: 654321

Valid for 30 seconds.

Ans 44.                  START

│

▼

Initialize ESP32

│

▼

Initialize PIR Sensor, Buzzer,

LEDs and Telegram

│

▼

Read Current Time Window

│

▼

Is Time Within Security Hours?

│             │

No            Yes

│             │

▼             ▼

Keep Monitoring   Read PIR Sensor

│

▼

Motion Detected?

│         │

No        Yes

│         │

▼         ▼

Continue Loop   Turn ON LED 1

Play Warning Beep

│

▼

Turn ON LED 2

Play Alarm Beep

│

▼

Turn ON LED 3

Play Urgent Beep

│

▼

Send Telegram Alert

Log Event with Time

│

▼

Repeat Loop

Code- Start

Initialize ESP32

Initialize PIR Sensor

Initialize Buzzer

Initialize Three LEDs

Initialize Telegram

Set Security Time Window

Loop

Read Current Time

If Current Time is Within Security Hours

Read PIR Sensor

If Motion Detected

Turn ON LED1

Play Warning Beep

Turn ON LED2

Play Alarm Beep

Turn ON LED3

Play Urgent Beep

Send Telegram Alert

Log Motion Event with Timestamp

Else

Turn OFF LEDs

Turn OFF Buzzer

End If

End If

Repeat

Sample serial monitor- Time,Motion,Alert Level,Status

22:01:15,Detected,Warning,LED1 ON

22:01:18,Detected,Alarm,LED2 ON

22:01:22,Detected,Urgent,LED3 ON

22:01:25,Detected,Telegram Sent

22:05:40,No Motion,System Monitoring

Sample telegram output

🚨 SECURITY ALERT 🚨

Motion Detected at Shop

Time: 22:01:25

Alert Level: URGENT

Please Check Immediately.

Ans 45.                  START

│

▼

Initialize ESP32

│

▼

Initialize DHT11, LDR, LCD,

Heater LED, Fan LED, Grow LED

│

▼

Read Temperature, Humidity & LDR

│

▼

Calculate Running Average of LDR

(Last 10 Readings)

│

▼

Is Temperature < 18°C?

│              │

Yes             No

│              │

▼              ▼

Heater ON       Is Temperature > 22°C?

│            │

Yes           No

│            │

▼            ▼

Fan ON      Heater & Fan OFF

│

▼

Is Humidity Between 60–70%?

│

▼

Check Light Intensity (LDR)

│

▼

Is Light Sufficient for 12 Hours?

│                 │

No                Yes

│                 │

▼                 ▼

Grow Light ON     Grow Light OFF

│

▼

Display Values on LCD

Publish Data to MQTT

│

▼

Wait 3 Seconds

│

▼

Repeat Loop

Start

Initialize ESP32

Initialize DHT11

Initialize LDR

Initialize LCD

Initialize Heater LED

Initialize Fan LED

Initialize Grow Light LED

Initialize MQTT

Loop

Read Temperature

Read Humidity

Read LDR Value

Calculate Running Average of Last 10 LDR Readings

If Temperature < 18

Turn ON Heater

Turn OFF Fan

Else If Temperature > 22

Turn ON Fan

Turn OFF Heater

Else

Turn OFF Heater

Turn OFF Fan

End If

If Light Level is Low

Turn ON Grow Light

Else

Turn OFF Grow Light

End If

Display Temperature

Display Humidity

Display Light Level

Publish Data to MQTT

Wait 3 Seconds

Repeat

Sample LCD display

Temp : 20°C

Humidity : 65%

Light : Normal

Heater : OFF

Fan : OFF

Grow Light : OFF

Sample serial monitor output

Time,Temp,Humidity,Light,Heater,Fan,GrowLight

10:00:00,19.5,64,420,OFF,OFF,OFF

10:00:03,17.2,63,300,ON,OFF,ON

10:00:06,22.8,66,520,OFF,ON,OFF

10:00:09,20.1,65,410,OFF,OFF,OFF

Ans 46.                  START

│

▼

Initialize ESP32

│

▼

Initialize HC-SR04, LEDs,

Buzzer, Wi-Fi & Dashboard

│

▼

Read Ultrasonic Distance

│

▼

Calculate Moving Average (10 Readings)

│

▼

Is Distance > 30 cm?

│             │

Yes            No

│             │

▼             ▼

NORMAL Level   Is Distance > 15 cm?

│            │

Yes           No

│            │

▼            ▼

WARNING Level   CRITICAL Level

│            │

Yellow LED + Medium Beep   Red LED + Fast Beep

│            │

└──────┬─────┘

▼

Send Alert via Wi-Fi/Bluetooth

│

▼

Update Live Chart on Dashboard

│

▼

Wait 2 Seconds

│

▼

Repeat Loop

Start

Initialize ESP32

Initialize HC-SR04

Initialize LEDs

Initialize Buzzer

Initialize Wi-Fi

Initialize Dashboard

Loop

Read Distance

Calculate Moving Average

If Distance > 30

Turn ON Green LED

Turn OFF Yellow LED

Turn OFF Red LED

Buzzer OFF

Level = NORMAL

Else If Distance > 15

Turn ON Yellow LED

Turn OFF Green LED

Turn OFF Red LED

Medium Beep

Level = WARNING

Else

Turn ON Red LED

Turn OFF Green LED

Turn OFF Yellow LED

Fast Beep

Level = CRITICAL

End If

Send Alert Message

Update Dashboard Graph

Print Distance and Level

Wait 2 Seconds

Repeat

Sample seriaql monitor

Time,Distance(cm),Water Level,Status

10:00:00,42,NORMAL,SAFE

10:00:02,28,WARNING,ALERT

10:00:04,12,CRITICAL,DANGER

10:00:06,10,CRITICAL,DANGER

Ans 47.                  START

│

▼

Initialize ESP32

│

▼

Initialize MAX30102/LDR, LCD,

LED and Data Log File

│

▼

Read Heart Rate & SpO₂

│

▼

Calculate 5-Reading Rolling Average

│

▼

Display Values on LCD

│

▼

Is Heart Rate <50 or >120?

│

┌────────┴────────┐

▼                 ▼

Yes               No

│                 │

▼                 ▼

Turn ON Red LED   Is SpO₂ <94%?

│

┌────────┴────────┐

▼                 ▼

Yes               No

│                 │

▼                 ▼

Turn ON Red LED    LED OFF

│

▼

Save Data to health_log.csv

│

▼

Wait 2 Seconds

│

▼

Repeat Loop

Start

Initialize ESP32

Initialize MAX30102 Sensor

Initialize LCD

Initialize Red LED

Initialize Data Log File

Loop

Read Heart Rate

Read SpO2

Calculate 5 Reading Rolling Average

Display Heart Rate on LCD

Display SpO2 on LCD

If Heart Rate < 50 OR Heart Rate > 120

Turn ON Red LED

Else If SpO2 < 94

Turn ON Red LED

Else

Turn OFF Red LED

End If

Save Heart Rate and SpO2 to health_log.csv

Wait 2 Seconds

Repeat

Sample serial monitor output

Time,Heart Rate,SpO2,Status

10:00:00,76,98,NORMAL

10:00:02,82,97,NORMAL

10:00:04,124,96,HIGH HEART RATE

10:00:06,78,92,LOW SPO2

10:00:08,79,98,NORMAL

Ans 48

Street Lighting System

### Sensors Needed

- LDR Sensor

- PIR Motion Sensor

### Microcontroller

- ESP32

### Connectivity

- Wi-Fi (MQTT)

### Data Flow

LDR + PIR Sensor → ESP32 → MQTT → Cloud → Dashboard

Approx. cost

ESP32 - ₹500

LDR - ₹50

PIR - ₹120

Relay - ₹100

LED - ₹50

Total ≈ ₹820

Start

Initialize ESP32

Initialize LDR

Initialize PIR

Initialize Relay

Loop

Read LDR

Read PIR

If Light is Low AND Motion Detected

Turn ON Street Light

Else

Turn OFF Street Light

End If

Send Status to Cloud

Repeat

# Air Quality Monitoring System

### Sensors Needed

- MQ-2 Gas Sensor

- DHT11

### Microcontroller

- ESP32

### Connectivity

- Wi-Fi (MQTT)

### Data Flow

MQ2 + DHT11 → ESP32 → MQTT → Cloud → Dashboard

### Approximate Cost

ESP32 - ₹500
MQ-2 - ₹180
DHT11 - ₹120
Total ≈ ₹800

### Pseudo Code

Start
Initialize ESP32
Initialize MQ2
Initialize DHT11
Loop
Read Gas Value
Read Temperature
Read Humidity
If Gas Value is High
Send Alert
End If
Upload Data to Cloud
Repeat

Ans 49.

## Security Vulnerabilities

| Vulnerability | Risk | Solution |
| --- | --- | --- |
| Hardcoded Wi-Fi SSID and Password | Credentials can be stolen if the code is shared. | Store credentials in config.h and exclude it using .gitignore. |
| Using HTTP instead of HTTPS | Data can be intercepted or modified by attackers. | Use HTTPS with SSL/TLS encryption. |
| Public MQTT Broker without Authentication | Anyone can publish or subscribe to messages. | Use MQTT username/password authentication and TLS. |
| No Validation of Incoming Commands | Attackers can send malicious commands to the device. | Validate all received commands before executing them. |
| No Data Encryption | Sensitive data is transmitted in plain text. | Encrypt communication using TLS/SSL. |

## Corrected Code 1 (Store Credentials Securely)

#include "config.h"
const char* ssid=WIFI_SSID;
const char* password=WIFI_PASSWORD;

config.h

#define WIFI_SSID "Your_WiFi_Name"
#define WIFI_PASSWORD "Your_WiFi_Password"

## Corrected Code 2 (Use HTTPS Instead of HTTP)

WiFiClientSecure client;
HTTPClient https;
https.begin(client,"https://example.com/api");
int httpCode=https.GET();

## Corrected Code 3 (Validate Commands)

String cmd=receivedCommand;
if(cmd=="ON"){
digitalWrite(LED,HIGH);
}
else if(cmd=="OFF"){
digitalWrite(LED,LOW);
}
else{
Serial.println("Invalid Command");
}

# Ans 50. Reflection & Learning Portfolio

During this IoT assignment, I learned how different sensors, microcontrollers, and communication technologies work together to build smart systems. I explored various IoT applications such as air quality monitoring, smart door locks, automated plant watering, greenhouse monitoring, flood detection, and healthcare monitoring. While designing these projects, I understood how sensors collect real-world data, how the ESP32 processes that data, and how information is transmitted using Wi-Fi, Bluetooth, HTTP, and MQTT protocols. I also learned the importance of cloud platforms, dashboards, and mobile applications for monitoring and controlling IoT devices remotely.

Another important learning outcome was understanding IoT security. I studied common security risks such as hardcoded passwords, unsecured HTTP communication, public MQTT brokers, and lack of data validation. I learned that secure communication using HTTPS, MQTT authentication, WPA3, and proper credential management are essential for protecting IoT devices from cyber attacks. This has helped me understand that security should always be considered while designing any connected system.

I also gained knowledge about designing algorithms, flowcharts, and system architecture before writing code. Creating flowcharts made it easier to understand the working of each project step by step. I learned how different sensors are selected based on the application, how threshold values are used for decision-making, and how alerts are generated using LEDs, buzzers, Bluetooth, or cloud notifications.

As an Electronics and Communication Engineering student, these IoT concepts are highly useful because they combine embedded systems, communication, networking, and programming. In the future, I would like to improve my skills by building complete hardware prototypes and connecting them to cloud platforms for real-time monitoring.

My next IoT project idea is a Smart Classroom Monitoring System. It would use sensors such as DHT11 for temperature and humidity, an MQ-2 sensor for air quality, an LDR for light intensity, and an ultrasonic sensor for occupancy detection. The ESP32 would collect sensor data and send it to a cloud dashboard using MQTT over Wi-Fi. Teachers and administrators could monitor classroom conditions through a mobile application, receive alerts when environmental conditions become unhealthy, and automatically control lights and fans to save energy. This project would improve student comfort, reduce electricity consumption, and make classrooms smarter and more energy efficient.

Overall, this assignment improved my understanding of IoT system design, sensor integration, wireless communication, cloud connectivity, and security. It also strengthened my problem-solving skills and gave me confidence to work on more advanced IoT projects in the future.